TABLE OF CONTENTS

I/O LIST STP 5 CMD MERDEKA

ITEM  DESCRIPTION	PAGE

- PURPOSE	3

- ABBREVIATIONS AND DEFINED TERMS	3

- I/O LIST	4

# PURPOSE

The purpose of this document is to provide a clear and comprehensive listing of all electrical Inputs and Outputs (IO) used within the system. It identifies each signal associated with field instruments, sensors, actuators, and control devices, along with their corresponding addresses and functional descriptions. This IO List is applicable for all Sediment Pond units and serves as a reference for configuration, integration, and troubleshooting across the entire system.

# ABBREVIATIONS AND DEFINED TERMS

The following terms are used within this document:

Table 2.1 – Defined Terms

| AG | Agitator |
| --- | --- |
| BQ | Dosing Pump |
| LS | Level Sensor |
| BA | Transfer Pump |
| FM | Flowmeter |
| PT | Pressure Transmitter |
| Q | Flowrate |

# I/O LIST

This section provides a concise listing of all electrical Input/Output (I/O) points used in the control system, including channel type, Modbus address, tag name, and signal function.

Table 3.O – I/O List STP 5 MERDEKA

| NO | Tag | Description | I/O |  | PLC |  |  |  |  | ADDRESS RS485 | SERVICE | REMARKS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | From | To | Signal | Type | Module | TAG | TERMINAL |  |  |  |
| 1 | EMG | Emergency Stop | MCC | MCC | DI | Dry | LOKAL 1 | I.00 | DI.1 | - | DISPLAY | - |
| 2 | T-TP-01A | Trip TP-01A | MCC | MCC | DI | Dry |  | I.01 | DI.2 | - | DISPLAY | - |
| 3 | T-TP-01B | Trip TP-01B | MCC | MCC | DI | Dry |  | I.02 | DI.3 | - | DISPLAY | - |
| 4 | T-TP-02A | Trip TP-02A | MCC | MCC | DI | Dry |  | I.03 | DI.4 | - | DISPLAY | - |
| 5 | T-TP-02B | Trip TP-02B | MCC | MCC | DI | Dry |  | I.04 | DI.5 | - | DISPLAY | - |
| 6 | T-TP-03 | Trip TP-03 | MCC | MCC | DI | Dry |  | I.05 | DI.6 | - | DISPLAY | - |
| 7 | T-TP-04A | Trip TP-04A | MCC | MCC | DI | Dry |  | I.06 | DI.7 | - | DISPLAY | - |
| 8 | T-TP-04B | Trip TP-04B | MCC | MCC | DI | Dry |  | I.07 | DI.8 | - | DISPLAY | - |
| 9 | T-TP-05A | Trip TP-05A | MCC | MCC | DI | Dry | LOKAL 2 | I.08 | DI.1 | - | DISPLAY | - |
| 10 | T-TP-05B | Trip TP-05B | MCC | MCC | DI | Dry |  | I.09 | DI.2 | - | DISPLAY | - |
| 11 | T-SM-01 | Trip SM-01 | STP | MCC | DI | Dry |  | I.10 | DI.3 | - | DISPLAY | - |
| 12 | T-dp-01 | Trip Dosing-01 | STP | MCC | DI | Dry |  | I.11 | DI.4 | - | DISPLAY | - |
| 13 | T-BL-01A | Trip Blower-01A | STP | MCC | DI | Dry |  | I.12 | DI.5 | - | DISPLAY | - |
| 14 | T-BL-01B | Trip Blower-01B | STP | MCC | DI | Dry |  | I.13 | DI.6 | - | DISPLAY | - |
| 15 | LS-101 | SIGNAL LS-101 | STP | MCC | DI | Dry |  | I.4 | DI.7 | - | DISPLAY | - |
| 16 | LS-102 | SIGNAL LS-102 | STP | MCC | DI | Dry |  | I.15 | DI.8 | - | DISPLAY | - |
| 17 | LS-103 | SIGNAL LS-103 | STP | MCC | DI | Dry | LOKAL 3 | I.16 | DI.1 | - | DISPLAY | - |
| 18 | LS-104 | SIGNAL LS-104 | STP | MCC | DI | Dry |  | I.17 | DI.2 | - | DISPLAY | - |
| 19 | AV-101 OP | AV-101 OPEN | STP | MCC | DI | Dry |  | I.18 | DI.3 | - | DISPLAY | - |
| 20 | AV-101 CL | AV-101 CLOSE | STP | MCC | DI | Dry |  | I.19 | DI.4 | - | DISPLAY | - |
| 21 | AV-102 OP | AV-101 OPEN | STP | MCC | DI | Dry |  | I.20 | DI.5 | - | DISPLAY | - |
| 22 | AV-102 CL | AV-101 CLOSE | STP | MCC | DI | Dry |  | I.21 | DI.6 | - | DISPLAY | - |
| 23 | PT-101 | PT-101 | STP | MCC | DI | Dry |  | I.22 | DI.7 | - | DISPLAY | - |
| 24 |  |  |  |  |  |  |  |  |  |  |  |  |

| DIGITAL OUTPUT |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Tag | Description | I/O |  | PLC |  |  |  | TERMINAL | ADDRESS RS485 | SERVICE | REMARKS |
|  |  |  | From | To | Signal | Type | Module | Address |  |  |  |  |
| - 1 | EMG | TP-01A | STP | MCC | DO | Dry | LOKAL 4 | Q.1 | DO1 | - | DISPLAY | - |
| - 2 | T-TP-01A | TP-01B | STP | MCC | DO | Dry |  | Q.2 | DO2 | - | DISPLAY | - |
| - 3 | T-TP-01B | TP-02 | STP | MCC | DO | Dry |  | Q.3 | DO3 | - | DISPLAY | - |
| - 4 | T-TP-02A | TP-03A | STP | MCC | DO | Dry |  | Q4 | DO4 | - | DISPLAY | - |
| - 5 | T-TP-02B | TP-03B | STP | MCC | DO | Dry |  | Q5 | DO5 | - | DISPLAY | - |
| - 6 | T-TP-03 | Agitator-01 | STP | MCC | DO | Dry |  | Q6 | DO6 | - | DISPLAY | - |
| - 7 | T-TP-04A | Dosing Pump-01 | STP | MCC | DO | Dry |  | Q7 | DO7 | - | DISPLAY | - |
| - 8 | T-TP-04B | Blower-01A | STP | MCC | DO | Dry |  | Q8 | D08 | - | DISPLAY | - |
| - 9 | T-TP-05A | Blower-01B | STP | MCC | DO | Dry | LOKAL 5 | Q9 | DO1 | - | DISPLAY | - |
| - 10 | T-TP-05B | AV-101 OPEN | STP | MCC | DO | Dry |  | Q10 | DO2 | - | DISPLAY | - |
| - 11 | T-SM-01 | AV-101 CLOSE | STP | MCC | DO | Dry |  | Q11 | DO3 | - | DISPLAY | - |
| - 12 | T-dp-01 | AV-102 OPEN | STP | MCC | DO | Dry |  | Q12 | DO4 | - | DISPLAY | - |
| - 13 | T-BL-01A | AV-102 CLOSE | STP | MCC | DO | Dry |  | Q13 | DO5 | - | DISPLAY | - |
| - 14 | T-BL-01B | ALARM | STP | MCC | DO | Dry |  | Q14 | DO6 | - | DISPLAY | - |
|  | AV-101 OP | AV-101 OPEN | STP | MCC | DO | Dry |  | Q15 | DO7 | - | DISPLAY | - |
|  | AV-101 CL | AV-101 CLOSE | STP | MCC | DO | Dry |  | Q16 | DO8 | - | DISPLAY | - |
|  | AV-102 OP | AV-102 OPEN | STP | MCC | DO | Dry |  | Q17 | DO1 | - | DISPLAY | - |
|  | AV-102 CL | AV-102 CLOSE | STP | MCC | DO | Dry | LOKAL 6 | Q18 | DO2 | - | DISPLAY | - |

| ANALOG INPUT |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Tag | Description | I/O |  | PLC |  |  |  | TERMINAL | ADDRESS RS485 | SERVICE | REMARKS |
|  |  |  | From | To | Signal | Type | Module | Address |  |  |  |  |
|  | PT-101 | PT-101 | STP | MCC | DO | Dry | LOKAL 6 | AI1+/- | AI1 | - | DISPLAY | - |
|  | LT-101 | LT-101 | STP | MCC | DO | Dry |  | AI2+/- | AI2 | - | DISPLAY | - |

| ANALOG OUTPUT |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Tag | Description | I/O |  | PLC |  |  |  | TERMINAL | ADDRESS RS485 | SERVICE | REMARKS |
|  |  |  | From | To | Signal | Type | Module | Address |  |  |  |  |
|  | TP-03 | TP-03 | STP | MCC | DO | Dry | LOKAL 6 | AO1+/- | AO1 | - | DISPLAY | - |
|  | TP-04A | TP-03A | STP | MCC | DO | Dry |  | AO2+/- | AO2 | - | DISPLAY | - |
|  | TP-04B | TP-03B | STP | MCC | DO | Dry |  | AO3+/- | AO3 | - | DISPLAY | - |

| SIGNAL MODBUS RS485 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Tag | Description | I/O |  | PLC |  |  |  | TERMINAL | ADDRESS RS485 | SERVICE | REMARKS |
|  |  |  | From | To | Signal | Type | Module | Address |  |  |  |  |
|  | FT-101 | FT-101 | MCC | MCC | - | - | - | - | - | - | DISPLAY | - |
|  | FT-102 | FT-102 | MCC | MCC | - | - |  | - | - | - | DISPLAY | - |
|  | FT-103 | FT-103 | MCC | MCC | - | - |  | - | - | - | DISPLAY | - |
|  | DO-101 | DO-101 | MCC | MCC | - | - |  | - | - | - | DISPLAY | - |